第三次作业：

公交IC卡刷卡数据分析

题目：

交通大数据是由交通运行管理直接产生的数据（包括各类道路交通、公共交通、对外交通的刷卡、线圈、卡口、GPS、视频、图片等数据）、交通相关行业和领域导入的数据（气象、环境、人口、规划、移动通信手机信令等数据），以及来自公众互动提供的交通状况数据（通过微博、微信、论坛、广播电台等提供的文字、图片、音视频等数据）构成的。

本课程给出了一个公交刷卡样例数据集，包含有交易类型、交易时间、交易卡号、刷卡类型、线路号、车辆编号、上车站点、下车站点、驾驶员编号、运营公司编号等。试导入该数据集并做分析，要求实现以下功能要求：

- 刷卡量统计：

分别计算早上7点前和晚上10点之后的公共交通上车刷卡量。

- 构造乘客搭乘站点数分析函数：

构造一个函数，计算不同线路乘客的平均公交搭乘站点数及其标准差。

- 计算高峰小时刷卡量和公交刷卡量的高峰小时系数（PHF5和PHF15）：

注：该题目是交通工程交通量统计的衍生。高峰小时系数的计算方法简述起来就是：高峰小时刷卡量与高峰小时内高峰时段的刷卡量扩大为1小时的刷卡量之比。高峰小时系数是用高峰小时刷卡量与高峰小时流率的比值表示的，其中分为5分钟计高峰小时系数和15分钟高峰小时系数。

高峰小时刷卡量:一天内的高峰期间连续60min 的最大刷卡量，单位为次/小时（此处简化为出现最大刷卡量的那个小时的刷卡量）
高峰X分钟流量：一般将高峰小时划分为 5min、6min、10min或15min的连续时段内统计刷卡量，此连续 5、6、10或15min所统计刷卡量中最大的那个时段，就是高峰小时内的高峰时段，其刷卡量即为高峰X分钟流量。（此处简化为出现最大刷卡量的那个X分钟时段的刷卡量）

5分钟计高峰小时系数计算公式：PHF5=高峰小时刷卡量/（12*高峰5分钟流量）

15分钟高峰小时系数计算公式：PHF15=高峰小时刷卡量/（4*高峰15分钟流量）。

- 分别构造线路类、司机类和车辆类：

将线路编号1101–1120的线路及其所对应的司机和车辆信息输出为20个txt文档，并保存到一个文件夹中。

- 分析搭载乘客情况：

确定服务乘客人次最多的10个司机、10条线路、10个站点和10台车辆。

- （附加问题）大家试试从数据集中还能发现什么？

代码案例：

import os
import pandas as pd
from collections import Counter


# 1. 刷卡量统计
def count_card_swipes(data):
    morning_swipes = data[data['交易时间'].dt.hour < 7]
    evening_swipes = data[data['交易时间'].dt.hour > 22]
    morning_count = len(morning_swipes)
    evening_count = len(evening_swipes)
    return morning_count, evening_count


# 2. 构造乘客搭乘站点数分析函数
def analyze_passenger_stops(data):
    all_routes = data['线路号'].unique()
    for route in all_routes:
        route_data = data[data['线路号'] == route]
        num_stops = abs(route_data['下车站点'] - route_data['上车站点'])
        mean_num_stops = num_stops.mean()
        std_num_stops = num_stops.std()
        print(f"线路号 {route} 的乘客平均公交搭乘站点数为 {mean_num_stops:.2f}，标准差为 {std_num_stops:.2f}")
    return
    # return avg_stops, std_stops


# 3. 计算高峰小时刷卡量和公交刷卡量的高峰小时系数（PHF5和PHF15）
def calculate_peak_hour_factor(data):
    PHV_list = []
    for i in range(0,24):
        PHV_list.append(len(data[data['交易时间'].dt.hour == i]))
    PHV = max(PHV_list)
    phv_index = PHV_list.index(max(PHV_list))
    peak_hour_swipes = data[(data['交易时间'].dt.hour >= phv_index) & (data['交易时间'].dt.hour <= phv_index+1)]
    #五分钟高峰流量
    phv5_list = []
    for j in range(0,11):
        phv5_list.append(len(peak_hour_swipes[(peak_hour_swipes['交易时间'].dt.minute >= j*5) & (
                peak_hour_swipes['交易时间'].dt.minute <= (j+1)*5)]))
    PHV5 = max(phv5_list)
    #15分钟高峰流量
    phv15_list = []
    for j in range(0, 3):
        phv15_list.append(len(peak_hour_swipes[(peak_hour_swipes['交易时间'].dt.minute >= j * 15) & (
                    peak_hour_swipes['交易时间'].dt.minute <= (j + 1) * 15)]))
    PHV15 = max(phv15_list)
    phf5 = PHV / (12 * PHV5)
    phf15 = PHV / (4 * PHV15)
    return PHV, phf5, phf15


# 4. 分别构造线路类、司机类和车辆类
class Route:
    def __init__(self, route_number, driver, vehicle):
        self.route_number = route_number
        self.driver = driver
        self.vehicle = vehicle


class Driver:
    def __init__(self, driver_number):
        self.driver_number = driver_number


class Vehicle:
    def __init__(self, vehicle_number):
        self.vehicle_number = vehicle_number


def export_route_info(data):
    routes = data[data['线路号'].between(1101, 1120)]
    os.mkdir('route_list')

    for route_number in range(1101, 1121):
        route_data = routes[routes['线路号'] == route_number]
        driver_numbers = route_data['驾驶员编号'].unique()
        vehicle_numbers = route_data['车辆编号'].unique()

        route = Route(route_number, driver_numbers, vehicle_numbers)

        with open(f'route_list/route_{route_number}.txt', 'w') as file:
            file.write(f'Route Number: {route.route_number}\n')
            file.write(f'Drivers: {", ".join(str(driver) for driver in route.driver)}\n')
            file.write(f'Vehicles: {", ".join(str(vehicle) for vehicle in route.vehicle)}\n')


# 5. 分析搭载乘客情况
def analyze_passenger_load(data):
    top_10_drivers = data.groupby('驾驶员编号').size().nlargest(10)
    top_10_routes = data.groupby('线路号').size().nlargest(10)
    top_10_stations = data.groupby('上车站点').size().nlargest(10)
    top_10_vehicles = data.groupby('车辆编号').size().nlargest(10)

    return top_10_drivers, top_10_routes, top_10_stations, top_10_vehicles


# 导入数据集
data = pd.read_csv('ICData.csv', parse_dates=['交易时间'])

# 调用函数进行分析
morning_count, evening_count = count_card_swipes(data)

pht, phf5, phf15 = calculate_peak_hour_factor(data)
export_route_info(data)
top_10_drivers, top_10_routes, top_10_stations, top_10_vehicles = analyze_passenger_load(data)

# 打印结果
print(f'要求1：\n早上7点前的刷卡量：{morning_count}')
print(f'晚上10点后的刷卡量：{evening_count}\n')
print(f'要求2：')
analyze_passenger_stops(data)
print(f'要求3：\n高峰小时刷卡量PHT：{pht}。公交刷卡量的高峰小时系数PHF5：{phf5}，PHF15：{phf15}\n')
print(f"要求5：\n分析搭载乘客情况:\n服务乘客人次最多的10个司机:{top_10_drivers}\n服务乘客人次最多的10条线路:{top_10_routes}\n"
      f"服务乘客人次最多的10个站点:{top_10_stations}\n服务乘客人次最多的10台车辆:{top_10_vehicles}")